CPU ARCHITECTURE :

Specifications

Main memory:

- Bootstrap memory : 0-127 : 128 bytes

- Instruction memory : 128 - 1023 : 896 bytes

- Data memory : 1024 - 4095 : 3.072 KB

Description: Memory is byte addressable (data type-uint8_t),  and  word size is 32 bits/4 bytes.

Registers:

General purpose registers: r0-r7 : all 4 word i.e. uint32_t

Number of registers : 8 registers

Special Purpose Registers:

Flag_register : 16 bit register that updates in case of Memory overflow with a value to

set the 12 bit.

Instruction Pointer: 32 bit register which holds the address of the next instruction to

be executed. It is incremented by 4 bytes after every instruction and thus points to

the next instruction.(Since instruction is 32 bits wide)

Arithmetic Logical Unit:

Performs logical operation on the data and provides it to the registers(To be

implemented in future)

Operations

1.   Load Operation

Instruction: lw register, (offset)memory address

Method Implemented: int loadreg(char* reg1,uint32_t* reg_num, int memory_addr );

- Method Name: loadreg

- Return type: int

- Register name: reg1 (r0-r7)

- Pointer to the Register: reg_num (pointer to reg1)

- Memory Address: memory_addr

Method Description: After taking input from user, 32-bit register is loaded with values that are present in 4 consecutive 8 bit/byte addressable memory locations. The final memory location is the result of addition of offset address as well as memory address.

2.       Store Operation

Instruction: sw register,(offset)memory address

Method Implemented: int storereg(char*reg1,uint32_t* reg_num, int memory_addr );

- Method Name: storereg

- Return type: int

- Register name: reg1 (r0-r7)

- Pointer to the Register: reg_num (pointer to reg1)

- Memory Address: memory_addr

Method Description: After taking input from user, values are loaded from the register into 4 consecutive 8 bit/byte addressable memory locations. The final memory location is the result of addition of offset address as well as memory address.

Instruction format

Instruction size : 4 bytes (opcode+register+offset+memory address)

The CPU implements a 32-bit instruction set architecture with Little-Endian format. Each memory related instruction (Load/Store) is designed as below with the following fields:

- Opcode: 8 bits (0x00→Load, 0x01→Store) This field specifies the type of the operation being performed in the instruction. The MSB byte of the instruction represent the opcode.

- Register: 4 bits (0x00-0x07 → r0-r7) This field represents the number corresponding to the actual register which is being used in the Load/Store operation. This field occupies the MSB nibble next to the opcode.

- Offset: 8 bits (0x00-0xff) This field specifies the offset amount of the memory address to be accessed. This takes the byte next to the register field in the instruction.

- Memory: 12 bits (0x00-0xfff) This field represents the address of memory from/to which a memory access should be performed. The width of this LSB field is chosen such that the instruction allows to access the memory available (i.e 4096 unique memory locations).

Screen Shots

- Initial Processor Start :